CPSC3300 – Computer Systems Organization
Homework #2 – Boolean Algebra and Adders

Due: 11:59PM Monday, February 10

Submit to Canvas

Total 100pts

- [20pts] Logical completeness

  - Show that you can use only two-input NAND gates to implement each of the following two-input logic functions, and draw the used NAND gates and wiring.

    - NOR function

    - XOR function

  - Show that you can use only two-input NOR gates to implement each of the following two-input logic functions, and draw the used NOR gates and wiring.

    - NAND function

    - XOR function

- [10pts] Show how to use 2-1 Muxes to build a 4-1 Mux. Draw the used 2-1 Muxes and the wiring, and mark the 4 inputs and 1 output for the resulting 4-1 Mux.

- [10pts] Demonstrate by means of truth tables whether the following identities are valid or not:

  - A+B+C=A+B+C

(O1)       (O2)           Not Valid

| Input |  |  | Output |  |
| --- | --- | --- | --- | --- |
| A | B | C | O1 | O2 |
| 0 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 |
| 0 | 1 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 0 | 0 |

  - A∙B+C=(A+C)∙(B+C)

(O1)         (O2)           Valid

| Input |  |  | Output |  |
| --- | --- | --- | --- | --- |
| A | B | C | O1 | O2 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 |

- [20pts] Prove the identity of each of the following Boolean equations, using algebraic manipulation:

  - (A+B)∙(A+B)∙(A+B)=A∙B

  - (AA+AB+BA+BB)∙(A+B)

  - (A+AB+BA)∙(A+B)

  - AA+ABA+BAA+AB+ABB+BAB

  - AB+ABB

  - AB+AB

  - AB

  - A∙B+B∙C+A∙B+B∙C=1

  - AB+BC+AB+BC

  - BC+C+BA+A

  - B1+B1

  - B+B

  - 1

- [20pts] For the Boolean function O1 and O2, as given in the following truth table:

| Input |  |  | Output |  |
| --- | --- | --- | --- | --- |
| x | y | z | O1 | O2 |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

  - List the minterms for a three-variable function with variables x, y, and z.

x’y’z’

x’y’z

x’yz’

x’yz

xy’z’

xy’z

xyz’

xyz

  - Express O1 and O2 in sum-of-product algebraic form.

    - O1: x’y’z’ + xy’z + xy’z’ + xy’z + xyz

    - O2: x’y’z + xy’z’ + xyz’ + xyz’

- [20pts] In class, we learned the implementation for a 4-bit carry lookahead adder. We can use the same idea and extend to build a 16-bit carry lookahead adder. Denote this implementation as a one-level carry lookahead adder.

In the textbook, Figure 8.6.3 shows a two-level implementation of a 16-bit carry lookahead adder. This adder uses 4-bit carry lookahead adders at the lower level, and uses a carry lookahead unit at the higher level.

Compare these two implementations and provide your explanation why the two-level implementation could be preferred.

- Due to the fact that each bit needs to take from every lower bit, the one level adder requires way more logic fates than the two-level, and is therefore less practical.